Table S1. Metadata of LA-MC-ICPMS U-Pb and Lu-Hf data used in this study

| Sample Preparation |  |
| --- | --- |
| Sample type/mineral | Detrital zircons |
| Sample preparation | Conventional mineral separation, 1 inch resin mount, 1 mm polish to finish |
| Imaging | Gatan Mono CL 4+ attached to Carl Zeiss SUPRA55SAPPHIR, 10 nA, 17 mm working distance |
| LA-ICPMS U-Pb analyses |  |
| Laboratory name | State Key Laboratory of Geological Processes and Mineral Resources, China University of Geosciences |
| Laser ablation system |  |
| Make, Model and type | Coherent, GeoLas 193 ArF excimer |
| Ablation cell and volume | In-house built low-volume cell, volume ca. 14 cm3 |
| Laser wavelength (nm) | 193 nm |
| Pulse width (ns) | 12 ns |
| Fluence (J cm-2) | 10 J cm-2 |
| Repetition rate (Hz) | 5 Hz |
| Ablation duration (s) | 45 s |
| Ablation pit depth / ablation rate | not available |
| Spot diameter (mm) nominal/actual | 32 mm |
| Sampling mode / pattern | Static spot ablation |
| Carrier gas | 100% He in the cell, N2 were introduced into the make-up gas via a T-connector |
| Cell carrier gas flow (l min-1) | ca. 0.80 L/min; fine-adjusted daily |
| ICP-MS Instrument |  |
| Make, Model and type | Agilent, 7500a quadrupole ICP-MS |
| Sample introduction | dry aerosol by laser ablation |
| RF power (W) | 1350W |
| Make-up gas flow (l min-1) | ca. 0.8 L/min; fine-adjusted daily |
| Detection system | Dual (pulse and analogue counting) |
| Masses measured | 29Si, 49Ti, 89Y, 91Zr, 139-175REE, 202Hg, 204(Hg+Pb), 206Pb, 207Pb, 208Pb, 232Th, 238U |
| Integration time per peak/dwell times (ms); quadrupole settling time between mass jumps | 6 ms for Si, Ti, Y, Zr, Nb, Hf, Ta, Hg and REE 15 ms for Pb, 10 ms for Th and U; settling time are allocated automatically by Agilent ICPMS software |
| Total integration time per output data point (s) | 0.124 s |
| ‘Sensitivity’ as useful yield (%, element) | not available for quadrupole ICPMS |
| IC Dead time (ns) | 30ns for EM of quadrupole ICPMS |
| Data Processing |  |
| Gas blank | 20 s on-peak zero subtracted |
| Calibration strategy | 91500 used as calibration standard, GJ-1 used as quality control for validation |
| Reference Material info | 91500 (Wiedenbeck et al. 1995) |
|  | GJ-1 (Jackson et al. 2004) |
| Data processing package used / Correction for LIEF | Commercial software ICPMSDataCal was intially developed by Prof. Yong-sheng LIU of GUG. LIEF correction assumes reference material and samples behave identically, namely standard-samples-standard bracketing. |
| Mass discrimination | not available |
| Common-Pb correction, composition and uncertainty | No common-Pb correction applied to the data. |
| Uncertainty level and propagation | Ages are quoted at 2s absolute, propagation is processed by the ICPMSDataCal software. Reproducibility and age uncertainty of reference material are propagated. |
| Quality control / Validation | GJ-1 – Weighted ave 206Pb/238U age = 600.1 ± 1.8 (2σ, MSWD = 0.6, n = 25) |
|  | Systematic uncertainty for propagation is 1.5% (1s). |
| Other information | ca. 2m sample line from ablation cell to torch and washout time was ca. 10 s to 0.1% of peak signal. |
| LA-MC-ICPMS Lu-Hf analyses |  |
| Laboratory and Sample Preparation |  |
| Laboratory name | State Key Laboratory of Geological Processes and Mineral Resources, China University of Geosciences |
| Laser ablation system |  |
| Make, Model and type | Lambda Physik, Gttingen, Germany, GeoLas2005 laser ablation system |
| Ablation cell and volume | HelEx II active 2-volume ablation cell, ~5 cm3 for small cell |
| Laser wavelength (nm) | 193 nm |
| Pulse width (ns) | 5 ns |
| Fluence (J cm-2) | 4 J cm-2 |
| Repetition rate (Hz) | 10 Hz |
| Ablation duration (s) | 40 s |
| Ablation pit depth / ablation rate | about 15μm |
| Spot diameter (mm) nominal/actual | 44 μm |
| Sampling mode / pattern | Static spot ablation |
| Carrier gas | 100% He in the cell, N2 were introduced into the make-up gas via a T-connector |
| Cell carrier gas flow (l min-1) | 0.85 L/min |
| MC-ICP-MS Instrument |  |
| Make, Model and type | Thermo Fisher Scientific, Germany, Neptune Plus MC-ICP-MS |
| Sample introduction | dry aerosol by laser ablation |
| RF power | 1270 W |
| Make-up gas flow (l min-1) | ca. 0.3-1.31 L/min |
| Detection system | Faraday cups |
| Masses measured | 171Yb, 173Yb, 174(Hf+Yb), 175Lu,176(Hf+Yb+Lu), 177Hf, 179Hf, 180(Hf+W), 182W |
| Integration time per peak/dwell times (ms); quadrupole settling time between mass jumps | 1s |
| Total integration time per output data point (s) | 70s |
| ‘Sensitivity’ as useful yield (%, element) | not available |
| IC Dead time (ns) | not available |
| Data Processing |  |
| Gas blank | 20 s on-peak zero subtracted |
| Calibration strategy | 91500 used as calibration standard, GJ-1 used as quality control for validation |
| Reference Material info | 91500 (Wu et al. 2006) |
|  | GJ-1 (Yuan et al. 2008) |
| Data processing package used / Correction for LIEF | Commercial software ICPMSDataCal was intially developed by Prof. Yong-sheng LIU of GUG. LIEF correction assumes reference material and samples behave identically, namely standard-samples-standard bracketing. |
| Mass discrimination | internal calibration by 179Hf/177Hf and 173Yb/171Yb |
| Uncertainty level and propagation | Ages are quoted at 2s absolute, propagation is processed by the ICPMSDataCal software. Reproducibility and age uncertainty of reference material are propagated. |
| Quality control / Validation | 91500 – Weighted ave 177Hf/176Hf = 0.282309 ± 0.000009 (2s, MSWD = 0.9, n = 6). GJ-1 – Weighted ave 177Hf/176Hf = 0.282013 ± 0.000006 (2s, MSWD = 1.1, n = 13) |
|  | Systematic uncertainty for propagation is 0.0015% (1s). |
| Other information | ca. 2m sample line from ablation cell to torch and washout time was ca. 1s to 1% of peak signal. |